DownLoad Station Preparation

Graham Tough (SWOC)

Run SIManager and choose the SI station option then Prepare then Training Mode.

On the right hand side of the  Prepare SI stations - Training mode  window, above the Write button, select the Directly option; use the magnet to switch on the master station and click the Read button.

In the Parameters section under Current contents you'll see the Working Time, Task and Code no. of the Master station displayed. (The Current time is irrelevant in this situation).

If any of these need to be altered use the 'white boxes' on the left.

Task should be set to Read SI cards

Code no, has to be in the range 1-31. (As far as I'm aware the number selected is irrelevant)

Working Time. This is the length of time the unit will stay on when activated by the magnet. It  can always be switched off earlier using the purple OFF dibber. I believe a default of 2 hours is recommended, but at a large event with high through-put this could be a nuisance. For such events 5 or 6 hours ought to be used. (Downside is that if left with this 5 /6 hour Working time and a magnet is inadvertently passed near the unit and it switches on, it will use 5/6 hrs of battery life before going off; the choice is yours!)

Format for altering Working Time is [h]h:mm

N.B. When any SI unit, including a Master Unit, is used in training mode the time-keeping in the unit starts at zero once the unit is triggered on by the magnet.